SEPARATION TECHNIQUES

BEHAVIOUR OBJECTIVES

At the end of this lesson, students should be able to:

- Explain reasons for separating mixtures and not compounds

- Outline at least 10 separation techniques

- Summarize the applications of the separation techniques

- Demonstrate simple separation techniques

Mixtures are as a result of physical change and so can be separated by physical means. The physical process used to separate mixtures is generally called separation technique.

The separation technique used to separate a particular mixture is based on the properties of the constituents in the mixture; such as the differences in solubility, boiling point, size, etc.

TYPES OF SEPARATION TECHNIQUES

- SIEVING: used to separate solid particles of different sizes. A sieve is used, smaller particles would pass through the sieve while larger solid particles will remain on the sieve.

APPLICATION: This method is used in mining and garri production.

- DECANTATION: used to separate insoluble solid from a liquid. The upper layer which is the clear liquid is decanted into a second container. This is a quick but inaccurate method.

APPLICATION: used in the processing of vinegar.

- FILTRATION: used to separate insoluble solid from a liquid using a filter paper. During filtration, the liquid portion passes through the filter paper, while the insoluble particle remains. The liquid that passes through the filter paper is called FILTRATE while the portion remaining on the filter paper is called the RESIDUE. Can be used to separate a mixture of chalk particles and water.

APPLICATION: used in water purification plants and breweries.

- EVAPORATION: used to separate dissolved solute from a solution by heating the solution to dryness.

Solution  solute + solvent

APPLICATION: used in salt- making industries.

- SUBLIMATION: is the direct conversion of solid to gas without passing through liquid state on the application of heat. Sublimation is used to separate a mixture that contains a constituent that sublimes. Examples of substances that sublimes; iodine, ammonium chloride, Sulphur, benzoic acid. A mixture of iodine and sodium chloride can be separated using sublimation.

APPLICATION: used in frozen food industry during freeze drying.

- DISTILLATION: used to recover a solvent from a solution. It is also used for separation of miscible liquids with widely different boiling points.

APPLICATION: used in gin distilleries.

- FRACTIONAL DISTILLATION: used to separate a mixture of two or more miscible liquids based on their boiling point. When the difference in their boiling point is less than 100oc, fractional distillation is strongly recommended.

APPLICATION: used in petroleum industry.

- SEPARATING FUNNEL METHOD: used to separate immiscible liquids. This separation is based on the differences in the densities of the constituents. The less dense will be on top and the denser below.

APPLICATION: used during solvent extraction in organic chemistry.

- CRYSTALLIZATION: used to separate pure crystals of salt which decompose easily on heating from its solution. When the purity of a product is important, crystallization is used.

APPLICATION: used in drug industries, sugar industries for purification.

- FRACTIONAL CRYSTALLIZATION: use to separate two or more solid solutes present in the same solution. The solutes to be separated must have different solubilities at different temperature.

APPLICATION: used in chemical engineering to recover saleable product from waste solutions.

- MAGNETIC SEPARATION: used to separate magnetic substance in the mixture of magnetic and non-magnetic substance.

APPLICATION: Used in steel and mining industries.

CRITERIA FOR PURITY FOR SUBSTANCES

Separation techniques are used to separate the constituents of a mixture. So, we can say a mixture is an impure substance. After conducting separation using any appropriate technique, we can then test for its purity.

Let’s talk about the criteria for purity;

- The melting point of a pure solid substance is fixed to be certain temperature.  e.g. naphthalene melts at 80oC, water (ice) melts at 0oC. If this temperature is affected, that indicate that there are impurities in the substance. For solids, the present of impurities lowers the melting point.  To ascertain the purity of a solid, the melting point should be determined.

- The boiling point of a pure liquid substance is fixed at a certain temperature. e.g. water boils at 100oC, ethanol boils at 78oC. If this temperature is affected, that indicates that there are impurities in the substance. For liquids, the present of impurities elevates the boiling point.  To ascertain the purity of a liquid, the boiling point should be determined.

- A pure substance gives one spot on paper chromatogram. This is used to check the purity of a coloured substances like pigments, dyes.

DRILL QUESTIONS

- Filtration is applied in the ______________?

- Production of petrol from petroleum

- Production of sodium chloride crystal from brine

- Separation of iodine and ammonium chloride crystals

- Purification of river water for town supply

- Two liquids A and B with boiling points 92oC and 93oC respectively can be best separated by?

- Boiling

- Distillation

- Evaporation

- Fractional Distillation

- A mixture of sand, ammonium chloride and sodium chloride is best separated by _________?

- Sublimation followed by addition of water and filtration

- Sublimation followed by addition of water and evaporation

- Addition of water followed by filtration sublimation

- Addition of water followed by crystallization

- Pure solvents are obtained by __________?

- Evaporation

- Extraction

- Condensation

- Distillation

- The purity of a solid sample can best be determined by its ___________?

- Boiling Point

- Conductivity

- Melting point

- solubility

ANSWER

- D

- D

- A

- D

- C