TRANISITION METALS

BEHAVIOUR OBJECTIVES

At the end of this lesson, students should be able to:

- classify transition metals [first and inner] with examples

- list at least 3 general properties of transition metals

- locate copper in the periodic table and electronic configuration

- explain the extraction of copper

- locate iron in the periodic table and electronic configuration

- mention the major ores of iron

- write at least 3 physical and chemical properties of iron

- list the conditions for rusting

- outline methods of prevention of rusting

Transition metals are elements found in the d-block of the periodic tale between group 2 and 13. They are associated with partially filled d-orbital and this is responsible for their unique characteristics. They occupy three rows, with ten elements in each row.  The first-row transition metals are from Scandium to Zinc, the second-row transition metals from Yttrium to Cadmium, and the third row is Lanthanum and then from Hafnium to Mercury.

There is another group called the inner transition metals, they are the Lanthanides and the Actinides, they are the f-block elements and have their valence electrons in the f-orbitals. So, we can conclude that the d-block is called "outer transition elements" and the f-block called "inner transition elements".

GENERAL PROPERTIES OF TRANSITION METALS

- Exhibits variable oxidation states.

- They possess the ability to form complex ions.

- They form coloured ions.

- They are paramagnetic.

- They form stable complexes.

COPPER

POSITION OF COPPER IN THE PERIODIC TABLE AND ELECTRONIC CONFIGURATION

Copper is the most electronegative element in the first-row transition elements. It is one of the less reactive metals, so it is lower on the electrochemical series.

Electronic Configuration: [Ar] 3d¹⁰ 4s¹

The electronic configuration of copper atom shows that it has completely filled 3d orbitals (3d10). This explains the unreactive and stable nature of copper.

ORES OF COPPER

- Copper pyrites (bornite), CuFeS2

- Copper glance (Chalcocite), Cu2S

- Cuprite, Cu2O

EXTRACTION OF COPPER

Copper is extracted from its sulphide ore (copper pyrites), CuFes2. The ore is concentrated by Froth Floatation (i.e. air blown to remove impurities).  After which, it is roasted in limited supply of air to oxidize the iron.

2CuFeS2(s) + 4O2(g)		→	Cu2S(s) + 2FeO(s) + 3O2(g)

Then, Silicon (IV)oxide is added and the mixture is heated in the absence of air to remove the iron (II)oxide in form of iron(II) trioxosilicate(IV) which floats on copper(I)sulphide as slang and is removed.

FeO(s) + SiO2(s)	→	FeSiO3(l)

The copper(I)sulphide that remains is then heated in a controlled amount of heat to produce copper with a purity of about 97-99%.

Cu2S(s) + O2(g)	→	2Cu(s) + SO2(g)

Further purification is obtained by electrolysis treatment of the copper. The cathodic copper formed is of high purity of about 99.9%.

USES OF COPPER

- Used in making electrical wires and cables.

- Used in making alloys, e.g. bronze, brass.

- Used in making cooking utensils.

- Used in ornamental works as it is not attacked by air.

SOME COMPOUNDS OF COPPER AND THEIR USES

| Chemical Formula | IUPAC Nomenclature | Uses |
| --- | --- | --- |
| CuO Cu (OH)2 CuSO4 Cu (NO3)2 CuCO3 | Copper(II) oxide Copper(II) hydroxide Copper(II) tetraoxosulphate(VI) Copper(II) trioxonitrate(V) Copper(II) trioxocarbonate(IV) | Used as pigment in ceramics. Used as fungicides Used in electroplating and in dyeing Used as catalyst Used as pigments in paints, varnishes. |

IRON

POSITION OF IRON IN THE PERIODIC TABLE AND ITS ELECTRONIC CONFIGURATION

Iron is a transition element, so it exhibits the properties of transition elements like forming coloured ions, existing in variable oxidation number, forming stable complex.

Electronic Configuration: [Ar] 3d⁶ 4s²

It is a constituent of the pigment’s haemoglobin in animals and chlorophyll in plants.

ORES OF IRON

- Haematite, Fe2O3

- Magnetite, Fe3O4

- Iron pyrite, FeS2

EXTRACTION OF IRON

It is extracted from its ore, haematite (oxide – Fe2O3), by reduction process in a blast furnace.

USES OF IRON

- Pig iron is used in making burners and cookers.

- Cast Iron is used in pipes, machines and automotive industry parts.

- Wrought Iron is used in making nails, iron rods, chains.

SOME COMPOUNDS OF IRON AND THEIR USES

| Chemical Formula | IUPAC Nomenclature | Uses |
| --- | --- | --- |
| Fe2O3 FeSO4 FeCl3 Fe2(SO4)3 | Iron(III) oxide Iron(II) tetraoxosulphate(VI) Iron(III) chloride Iron(III) tetraoxosulphate(VI) | Used as mild abrasive for polishing jewellery Used in making inks, tonics and pigments Used as flocculant sewage treatment Used as a soil conditioner |

PHYSICAL PROPERTIES OF IRON

- Silvery metal with high tensile strength

- Good conductor of heat and electricity.

- Malleable and ductile.

- Can be magnetized easily.

CHEMICAL PROPERTIES OF IRON

- Iron rusts on exposure to moist air

  - 4Fe(s) + 3O2(g) + 2xH2O(l)	→	2Fe2O3.xH2O(s)

- Red-hot iron fillings react with steam to give triiron tetraoxide and hydrogen

  - 3Fe(s) + 4H2O(g)	↔	Fe3O4(s) + H2(g)

- Iron reacts with acid to give its corresponding iron(ii)salt

  - Fe(s) + 2HCl(aq)	→	FeCl2(aq) + H2(g)

RUSTING OF IRON

Corrosion of iron by slow oxidation in the presence of air and moisture. When iron is exposed to moist air, it rusts and forms the brown-hydrated iron (III) oxide.

4Fe(s) + 3O2(g) + 2xH2O(l)	→	2Fe2O3.xH2O(s)

Oxygen and moist are the Condition of Rusting.

METHODS OF PREVENTION OF RUSTING

- By Coating: the application of a protective layer such as oil, grease, paint to serve as a barrier between the surface and the corroding agents; water and air.

- By galvanizing: carried out by electrolysis. Dipping the iron into a molten solution of a metal higher than it in the activity series, such as Al, Zn to prevent it from rusting.

- By alloying: mixed with other elements. e.g. stainless steel, steel.

DRILL QUESTIONS

- Rusting of iron is the slow process of …………………..?

- Oxidation

- Reduction

- Hydration

- Precipitation

- The correct electronic configuration Copper is ………............?

- [Ar] 3d9 4s2

- [Ar] 3d¹⁰ 4s¹

- [Ar] 4s2 3d9

- [Ar] 4s¹ 3d¹⁰

- Which of the following is not a characteristic of transition metals?

- Forms coloured ions

- Paramagnetic

- Forms colourless ions

- Forms stable complexes

- What is the formula of haematite?

- Fe3O4

- FeSO4.7H2O

- FeCl3

- Fe2O3

- The first transition element is ……………….?

- Copper

- Nickel

- Scandium

- Vanadium

ANSWER

- A

- B

- C

- D

- C